FLEXFORM  CABLE ENGINEERING

TECHNICAL DATASHEET

© All rights reserved.

All contents of this document are our property and any copy or divulgation is not allowed without our written authorisation.

Braided Double Insulated FEP/FEP Triple Twisted Flex Product Range: BTFF

| CABLE CONSTRUCTION 1. Viscose Rayon braid 2. R5F type FEP insulation 3. 180 °C transparent FEP outer insulation 3 4. Electrolytic tinned flexible concentric rope copper conductor | APPLICATIONS This product is designed for use as a pendant drop or table and floor lamps. Approval for rise and fall applications should be confirmed with ourselves prior to adoption. SCHEMATIC DRAWING 3 core construction CE |
| --- | --- |

Characteristics

| Voltage Rating (U₀/U): | 300 / 500 Vac |
| --- | --- |
| Test Voltage: | 2500 V |
| Temperature Range: | -55 °C ÷ 180 °C |
| Short-circuit Temperature: | 160 °C |
| Minimum Bending Radius: | 14 x Overall Diameter |

| Size Conductor [mm²] | Conductor Stranding [Nº x mm] | Electrical Resistance [Ω/Km] |
| --- | --- | --- |
| 0.75 | 19 x 0.224 | ≤ 26.7 |

| Flexform Product Code | Size Conductor [mm²] | Outer Diameter [mm] | Thickness of Jacket [mm] |
| --- | --- | --- | --- |
| BTFF-403 | 3 x 0.75 | 5.6 ± 0.5 | ≥ 0.60 |

All versions of this cable can be ordered with a plain or patterned viscose rayon textile braid, available in over 60 colours. Please contact us for a list of available colours.

| Cable Standards | Low Voltage European Directive No. 2014/35/UE. |
| --- | --- |
| Stranding of Conductor | Electrolytic tinned flexible concentric rope copper conductor. |
| Inner Insulation Outer Insultation | R5F type FEP conforming to EN-50363-0 requirements. R5F type FEP conforming to EN-50363-0 requirements. |
| Cable Assembly | The cores are braided in viscose rayon before being twisted together. |

Core Identification		3 Core: Brown  ●  Blue  ●  Green/Yellow  ●

| Revision Date: 21/07/2026 | Issue No. 1.3 | Page: 2 of 2 |
| --- | --- | --- |

In order to improve our products, the information contained in this technical datasheet can be changed without notice. Please check periodically with our office, the date and issue number.

FLEXFORM   ☎ +44 028 9079 2155    ✉ info@flexform.co.uk    ⌂ www.flexform.co.uk